UNIT-1

1. Define open channel

2. What are the classifications of flow in an open channel?

3. Define steady flow and unsteady flow

4. Define Uniform flow and Non-Uniform flow.

5. What is rapidly varied flow?

6. What is gradually varied flow?

7. What is laminar and turbulent flow?

8. What is TRANSITION state?

9. Give a brief note on Sub-critical, Critical, Super critical flow.

10.Give the formula relating to velocity and discharge in chezy’s formula

11.Give the BAZIN, GANGUILLET-KUTTER, MANNINGS formulas for chezy’s constant.

12. Give the formula for total energy.

13. Define specific energy.

14. How do you calculate specific energy?

15. What is specific energy curve?

16. Define critical depth.

17. What is critical velocity?

18.Represent minimum specific energy in terms of critical depth.

19. What is critical flow?

20. Define streaming flow. What it is otherwise called as?

UNIT-2

1.When does a super critical flow occur?

2. What is alternate depth?

3. Comparison between open channel flow and pipe flow

4. What are the types of channels?

5. What are regimes of flow?

6. What is wide open channel?

7.What do you mean by velocity distribution in an open channel?

8. What are the applications of specific energy?

9.What are the factors affecting Manning’s roughness coefficient?

10.What is top width, wetted area, wetted perimeter, Hydraulic mean radius, hydraulic depth?

11.Find the diameter of a circular sewer pipe which is laid at a slope of 1 in 8000 and carries a discharge of 800 litres/second.when flowing half full. Take the value of Manning’s N=0.02

12	i) Find the specific energy of flowing water through a rectangular channel of width 5m when the discharge is 10 m3/s and depth of water is 3m.

(ii) Find the critical depth and critical velocity of water flowing through a rectangular channel of width 5m, when discharge is 15m3/s.

13 The discharge of water through a rectangular channel of width 8m,is 15 m3/s when depth of flow of water is 1.2 m.Calculate:Specific energy of flowing water, Critical depth and critical velocity, Value of minimum specific energy.

14	i) The specific energy for a 5m wide rectangular channel is to be 4Nm/N.If the rate of flow of water through the channel is 20 m3/s.Determine the alternate depth of flow.

ii) Derive the froude number.

15 Condition for maximum discharge for given value of specific energy.

16	i) Find the velocity of flow and rate of flow of water through a rectangular channel of 6m wide and 3m deep, when it is running full. The channel is having bed slope as 1 in 2000. Take chezy’s constant C = 55.

ii) Find the slope of the bed of a rectangular channel 5m when depth of water is 2m and rate of  low is given as 20 m3/s. Take chezy’s constant, C = 50.

17) A flow of water of 100 lts/sec flows down in rectangular flume of width 600mm and having adjustable bottom slope. If chezy’s constant C is 56, find the bottom slope necessary for uniform flow with a depth of flow of 300mm. Also find the conveyance K of the flume.

18) Find the discharge through a trapezoidal channel of width 8m and side slope of 1 horizontal to 3 vertical. The depth 20 of flow of water is 2.4m and value of Chezy’s constant, C = 50. The slope of the bed of the channel is given 1 in 4000.

19.Find the bed slope of trapezoidal channel of bed width 6m, depth of water 3m and side slope of 3 horizontal to 4 vertical, when the discharge through the channel is 30 m3/s. Take Chezy’s Constant, C= 70 20

i) Find the discharge of water through the channel shown in the fig. Take the value of Chezy’s constant = 60 and slope of the bed as 1 in 2000.

ii) Find the rate of flow of water through a V- Shaped channel as shown in the fig. Take the value of C = 55 and slope of the bed 1 in 2000

UNIT-3

- Define hydraulic machine.

- Define impulse momentum principle

- Define momentum correction factor

- State the principles of jet propulsion

- Expression foe efficiency of jet propulsion

- Define moment of momentum

- Define impact of jet

- Give the expression for force exerted by jet on stationary vertical plate held normal to the jet

- Expression for force exerted by jet of water on moving inclined plate in the direction of moving plate

- Expression of force and work done for series of vanes

- Expression for max efficiency of series of vanes

- Expression for efficiency of radial curved vane

- Expression for angle of swing of a vertical hinged plate

- A horizontal jet of water 8m issues with a velocity of 20m/s and strikes a fixed vertical plate. Find the force exerted by the jet on the plate and also the work done.

- A jet of water 15cm in dia strikes at the centre of curved symmetric stationary plate with a vel of 15m/s. After striking the jet deflected through an angle of 120 degree. Determine the force exerted on the plate

- Expression of force and work done per second by the jet when it strikes the inclined plate moving in the direction of jet

- Expression for force exerted by a jet striking the curved plate at one end tangentially when the plate is symmetrical

- Efficiency of wheel

- Explain Significance of momentum equation

- Explain  Application of momentum equation

Unit-4

1) Draw a layout of hydroelectric power plant and explain its working briefly.

2) Give the classification of hydraulic turbines

3) State the function of draft tube and draw the neat sketches of draft tube

4) Differentiate impulse turbine with reaction turbine.

5) Differentiate Francis turbine with Kaplan turbine

6) Explain the working of Pelton wheel turbine.

7) A Pelton wheel has mean bucket speed of 30 m/s with a jet of water flowing at the rate of 1cubic meter per sec under a head of 250m. The bucket deflect jet through angle of 170 degree. Calculate power developed and the efficiency of the turbine. Assume Coefficient of velocity= 0.98.

8) A Francis turbine has external and internal diameters as 1m and 0.6m respectively . The hydraulic efficiency of turbine is 90% when the head on the turbine is 36m . The velocity of flow at outlet is 2.5m/sand discharge at outlet is radial. If the vane angle at outlet is 15degree and width of the wheel is 100mm at inlet and outlet. Determine (i) Guide blade angle (ii)speed of turbine

10) Explain the construction and working of centrifugal pump.

11) Explain submersible pump with neat sketch.

12) Explain jet pump with neat sketch.

13) What did you understand by the term cavitation? How it can be avoided?

14) Describe the principle and working of a reciprocating pump with neat sketch.

15) Differentiate between centrifugal pump and reciprocating pump.

16) Mention the applications of reciprocating pump.

17) Explain the working of double acting reciprocating pump

18) A jet of water 50mm in dia. under constant head of 50m impinges on a fixed blade normally. Find force exerted by jet if coefficient of velocity is 0.95.

19). Draw neat sketch for impact of jet on a moving vertical flat plate and write the formula to determine the work done.

20). A jet of water 50mm in dia. under constant head of 50m impinges on a fixed blade normally. Find force exerted by jet if coefficient of velocity is 0.95. 2. Draw neat sketch for impact of jet on a moving vertical flat plate and write the formula to determine the work done.

UNIT-5

1. Define pump.

2. What is meant by positive and non-positive displacement pump?

3. What is meant by fixed displacement and variable displacement pump?

4. What is the classification pump?

5. What is meant by a balanced vane pump?

6. Define volumetric efficiency of a pump.

7. Define mechanical efficiency of a pump.

8. Define overall efficiency of a pump.

9. Draw the graphical symbols of the following pumps

| i) Fixed displacement | ii) Variable displacement |  |  |
| --- | --- | --- | --- |
| 10. Give an expression for efficiency of draft tube. 11. What is meant by performance characteristics curves of turbine |  |  |  |  |  |

12. Define motor.

13. What is semi-rotary actuator?

14. List the application of semi-rotary actuator?

15 define degree of reaction

16 State the definition of gross head

17. What are the types of hydraulic cylinder?

18. Define the jet ratio

19 what are unit quantities

20. Define the propeller turbiune