VIDYA JYOTHI INSTITUTE OF TECHNOLOGY

Strength of Materials II

Unit I

Torsion of Circular Shafts

SHORT ANSWER QUESTIONS (To be done in 2nd copy)

- What is theory of pure torsion?

- Derive torsion equation T/J = q/r = Gθ/l

- Define Shaft.

- What are the assumptions made in the theory of pure torsion?

- What is torsional moment of resistance?

- Define polar section modulus.

- Explain power transmitted by shaft in terms of N (rpm) and f (frequency in Hz)

- Write down formula for major and minor principal stress for combined bending and torsion and end thrust?

- What are the different types of springs, explain with neat sketch.

- How to calculate deflection of close coiled helical spring.

- What is springs in series, explain with neat sketch.

- What is springs in parallel, explain with neat sketch.

- What is leaf spring?

- What is the maximum torque transmitted by circular shaft.

- Derive expression for torque in terms of polar moment of Inertia.

- Write down the formula for torque transmitted by a hollow circular shaft.

- Find the maximum shear stress induced in a solid circular shaft of diameter 15cm when the shaft transmits 150kW power at 180 rpm.

- What is strength of a shaft and torsional rigidity?

- A closely coiled helical spring is to carry a load of 1kN. Its mean coil diameter is to be 10 times that of wire diameter. Calculate these diameters if the maximum shear stress in the material of the spring is to be 90 N/mm2 (Ans: 16.82cm and 1.68cm)

- Prove that the central deflection of the laminated spring is given by

VIDYA JYOTHI INSTITUTE OF TECHNOLOGY

Strength of Materials II

Unit I

Torsion of Circular Shafts

LONG ANSWER QUESTIONS (To be done in 2nd copy)

1. Derive an expression for torque transmitted by a hollow circular shaft.

2. Derive an Expression for strain energy stored in a body due to torsion.

2. Two shafts of the same material and of same lengths are subjected to the same torque, if the first shaft is of a solid circular section and the second shaft is of hollow circular section, whose internal diameter is 2/3 of the outside diameter and maximum shear stress developed in each shaft is same, compare the weights of the shaft.

3. A hollow shaft of external diameter 120mm transmits 300kW power at 200rpm. Determine the maximum internal diameter if the maximum stress in the shaft is not to exceed 60N/mm2.

4. The maximum allowable shear stress in a hollow shaft of external diameter equal to twice the internal diameter is 80N/mm2. Determine the diameter of the shaft if it is subjected to a torque of 4X106 Nmm and a bending moment of 3X106 Nmm.

5. A solid shaft of diameter 80mm is subjected to a twisting moment of 8MN mm and a bending moment of 5 MN mm at a point. Determine:

i) Principal stresses and

ii) Position of the plane on which they act.

6. A soild circular shaft of 10cm diameter of length 4m is transmitting 112.5kW power at 150r.p.m. Determine: (i) the maximum shear stress induced in the shaft, (ii) Strain energy stored in the shaft. Take C= 8X104 ZN/mm2

7. A leaf spring carries a central load of 3000N. The leaf spring is to be made of 10 steel plates 5cm wide and 6mm thick. If the bending stress is limited to 150 N/mm2. Determine:

(i) Length of the spring

(ii) Deflection at the centre of the spring. Take E=2x105 N/mm2

8.  A close coiled helical spring of 10cm mean diameter is made up of 1cm diameter rod and 20 turns. The spring carries an axial load of 200N. Determine the shearing stress. Taking the value of modulus of rigidity = 8.4X104 N/mm2, determine the deflection, determine the deflection when carrying this load. Also calculate the stiffness of the spring and the frequency of free vibration for a mass hanging from it.

VIDYA JYOTHI INSTITUTE OF TECHNOLOGY

CIVIL ENGINEERING DEPARTMENT

B.TECH II YEAR- SEM II

Strength of Materials (Torsion of Shafts)

Q.1. For a solid or a hollow shaft subjected to a twisting moment T, the torsional shearing stress t at a distance r from the centre will be

(a) t = Tr/J  (b) t = Tr  (c) t = TJ/r  (d) none of these

where J is second moment of area.

Q.2. A hollow prismatic beam of circular section is subjected to a torsional moment. The maximum shear stress occurs at

(a) inner wall of cross section   (b) middle of thickness  (c) outer surface of shaft  (d) none of these (AMIE Winter 1997)

Q.3. A solid shaft has diameter 80 mm. It is subjected to a torque of 4 KNm. The maximum shear stress induced in the shaft would be

(a) 75/p N/mm2    (b) 250/p N/mm2   (c) 125/p N/mm2  (d) 150/p N/mm2 (IES 2001)

Q.4. If in a bar after twisting moment T has been applied, a line on surface is moved by an angle g then shearing moment will be

(a) t/g   (b) g   (c) g/t  (d) none of these

Q.5. Shear modulus G is given by

(a) G = t/g   (b) G = g/t  (c) G = Tg/t (d) G = T/g

Q.6. A shaft of length L is subject to a constant twisting moment T along its length L, then angle q through which one end of the bar will twist relative to other will be

(a) T/g   (b) T/GJ  (c) GJ/TL  (d) TL/GJ

Q.7. A circular shaft subjected to torsion undergoes a twist of 1 0 in a length of 120 cm. If the maximum shear stress induced is limited to 1000 kg/cm2 and if modulus of rigidity G = 0.8 x 106 then the radius of the shaft should be

(a) p/8  (b) p/27  (c) 18/p (d) 27/p (IES 1993)

Hint: t/r = Gq/l

Q.8. At fully plastic twisting moment

(a) only fibres at surface are stressed to yield point in shear (b) fibres at centre are stressed to yield point in shear

(c) all fibres are stressed to yield point in shear (d) none of these

Q.9. The relationship among twisting moment(T) acting on a rotating shaft, power in watt(W), and angular velocity in radian per second(w) will be

(a) T = W/w   (b) W = Tw   (c) W = T/w (d) none of these

Q.10. A shaft turns at 150 rpm under a torque of 1500 N-m. Power transmitted is

(a) 15p kW   (b) 10p kW (c) 7.5p kW (d) 5p kW (IES 1999) Hint: P = 2pNT/60

Answers:

1. a

2. c

3. c

4. b

5. a

6. d

7. d

8. c

9. b

10. c