VIDYA JYOTHI INSTITUTE OF TECHNOLOGY

Strength of Materials II (A40114)

Unit III

Beam Columns and Direct and Bending Stresses

SHORT ANSWER QUESTIONS (To be done in 2nd copy)

1. What is beam column?

2. In beam column, how to express combined bending and direct stresses.

3. Justify, how σmin is negative when stress along one side is tensile (Show relation in terms of relation between σ0 and σb)

4. Justify, how σmin is zero there will be no tensile stress along width of column (Show relation in terms of relation between σ0 and σb

5. Justify, how σmin is positive when there will be only compressive along the width of the column (Show relation in terms of relation between σ0 and σb

6. Define kernel of the section.

7. What is middle quarter rule for circular section?

8. What are the conditions of stability for dams and walls?

9. Show the relation about overturning of structure about toe can happen?

10. When there will be no sliding of the structures?

11. For a hollow circular chimney how to calculate weight of the shaft and wind pressure acting?

12.What is a formula for base earth pressure in terms of ‘w’ and ‘W’ (Hint: Rankine’s theory of earth pressure)

13. Define angle of repose?

14. Write down the formula of eccentricity for rectangular dam and trapezoidal dam.

15. Write down the formula for maximum and minimum stresses at the base of a dam having water face vertical.

16. Write down the formula for maximum and minimum stresses at the base of a dam having water face inclined.

17. What is the difference between a dam and a retaining wall.

18. Write down the formula for Thrust due to water on the vertical side of a dam.

19. What do you mean by stability of dam? What are the different conditions under which dam is going to fail?

20. What are the assumptions made in Rankine’s theory of earth pressure?

21. A strut, 30mm ɸ and 2.2m long, is hinged at both ends. It carries UD loads of 60 N/m in addition to an axial thrust of 8000N. Calculate the maximum stress. E=200 GPa.

22. If in the above example if strut carries point load of 150N instead of the UD load, find the maximum stress.

23. Write down secant’s formula.

LONG ANSWER QUESTIONS (To be done in 2nd copy)

1. In the rectangular section as shown in fig., the compressive load P=80kN is applied as shown. Find the stresses at each corner and draw the stress diagram. Find the position of the load in the same quadrant for the opposite corner to have zero stress.

Y

D

C

P

X

200mm

B

A

400mm

80mm

25mm

2. Find the kern of the I section shown in Figure.

Consider flanges of size 300mm, web 100mm, Height  500mm (assume data if not given)

3.  A rectangular column of width 200mm and of thickness 150mm carries a point load of 240kN at an eccentricity of 10mm as shown in Fig. Determine the maximum and min. stresses on the section.

4.  Draw neat sketch of kernel of the following cross section: (Bansal, 462)

(i) Rectangular section 200mm X 300mm

(ii)  Hollow circular cylinder with external diameter=300mm and thickness =50mm

(iii) Square with 400 cm2 area