Vidya Jyoti Institute of Technology

Structural Analysis I (A40115)

Unit II- Energy Theorems and Three Hinged Arches

(Refer Class notes and Vazirani and Ratwani)

- Short answer questions (2 to 3 Marks- Answer should be written in maximum Half page approximately)

- What is an arch? Explain.

- Distinguish between two hinged and three hinged arches?

- Give the equation for a parabolic arch whose springing is at different levels?

- State Eddy‟s theorem as applicable to arches?

- Explain the effect of temperature on the horizontal thrust of a two hinged arch subjected to a system of vertical loads?

- Indicate the positions of a moving point load for maximum negative and positive bending moments in a three hinged arch.

- Write down the expressions for radial shear and normal thrust in a three hinged parabolic arch?

- Define radial shear and normal thrust?

- Mention the examples where arch action is usually encountered?

- What is a linear arch?

- What is the degree of static indeterminacy of a three hinged parabolic arch?

- Under what conditions will the bending moment in an arch be zero throughout?

- In a parabolic arch with two hinges how will you calculate the slope of the arch at any point?

- What are the types of arches according to their shapes?

- What are the types of arches according to the support conditions?

- Explain strain energy in linear elastic system.

- Give expression for strain energy due to axial load, bending moment and shear forces.

- What is Castiglione’s first theorem?

- What is Unit load method?

- Explain deflection of statically determinate bent frames.

- Long Answer  Questions.

- A three hinged parabolic arch of span 100m and rise 20m carries a uniformly distributed load of 2KN/m length on the right half as shown in the figure.

the maximum bending moment in the arch.

- A three hinged parabolic arch, hinged at the crown and springing has a horizontal of 15m with a central rise of 3m. If carries a udl of 40kN/m over the left hand of the span. 3m. Calculate normal thrust, radial shear and bending moment at 5m from the left hand hinge.