Vidya Jyoti Institute of Technology

Structural Analysis I (A40115)

Unit V- Moving loads and Influence lines

(Refer Class notes and Vazirani and Ratwani)

1. Short answer questions (2 to 3 Marks- Answer should be written in maximum Half page approximately)

- Define Unit load, rolling load.

- What do you mean by influence line diagram?

- State Muller Breslau‟s principle?

- Draw influence lines for support reactions in a simply supported beam?

- What do you understand by an influence line for bending moment?

- When a series of wheel loads move along a girder, what is the condition for getting maximum bending moment under any one point load?

- Draw a qualitative influence line diagrams for the support reactions of a simply supported beam of span L?

- What is meant by absolute maximum bending moment in a beam?

- What is the absolute maximum bending moment due to a moving udl longer than the span of a simply supported beam?

- Where do you get rolling loads in practice?

- Name the type of rolling loads for which the absolute maximum bending moment occurs at the midspan of a beam?

- Where do you have the absolute maximum bending moment in a simply supported beam when a series of wheel loads cross it?

- State the location of maximum shear force in a simple beam with any kind of loading?

- What is meant by maximum shear force diagram?

- State Maxwell-Betti‟s theorem?

- Write down steps to draw influence line for forces in members of Pratt trusses.

- Write down steps to draw influence line for forces in members of Warren trusses.

- What is focal length?

- Give example of moving load in practice?

- What is a importance of Influence lines diagram?

2. Long answer questions (Questions with 5 to 10 marks)

- A single rolling load of 100 kN moves on a girder of span 20m. (a) Construct the influence lines for (i) Shear force and (ii) Bending moment for a section 5m from the left support. (b) Construct the influence lines for points at which the maximum shears and maximum bending moment develop. Determine these maximum values.

ii. Derive the influence diagram for reactions and bending moment at any section of a simply supported beam. Using the ILD, determine the support reactions and find bending moment at 2m, 4m and 6m for a simply supported beam of span 8m subjected to three point loads of 10kN, 15kN and 5kN placed at 1m, 4.5m and 6.5m respectively.

iii. Two concentrated rolling loads of 12 kN and 6 kN placed 4.5 m apart, travel along a freely supported girder of 16m span. Draw the diagrams for maximum positive shear force, maximum negative shear force and maximum bending moment. Draw Bending moment diagram for the propped cantilever loaded as shown in Figure. The supports A and B remain at the same level after the load. W=10kN, l=5m,